HANGAR OPS NOTES                             NR 06/04/08

Hangar Rules

No fuel transfer or fuel storage in hangar. (Ref: Massport rules.)

No engine start in hangar;  no propwash into open hangar door.

Keep hangar picked up. Shovel snow promptly after snowstorms.

Before Rolling Out

Hanger door fully up, or tail will not clear.

Don’t bump head on partially open door (sharp sheet metal edges).

Ck for yoke or panel placards, daybook notes, tach-time on ticket, VOT notes.

First person to fly each month checks tire pressures. Don’t mislay valve extender.

If preflighting in the hangar, have enough light, really see, don’t guess.

Use clean spout extender to add oil. (Record oil use via t.s. on line, also daybook.)

Use clean (i.e. non-gritty) wipers on canopy and windows.

If needed (and within W&B), take oil, sunscreens, tiedowns, other aux stuff.

When Rolling Out

If an airplane is on one end of the towbar, a (qualified) hand must be on the other.

After clear of hangar, aim aircraft to the left (ccw circulation around buildings).

Closely supervise any helpers:  have them push on the struts near att. points.

Keep them far away from the prop. Don’t let them dent the aluminum surfaces.

Watch out for slippery ramp conditions – ice, snow, turkey-shit.

Don’t track snow, mud, sand, droppings, etc. into the plane.

Best not to roll over drain grates or stop with plane doors directly over them.

Use strainer-cup to drain sumps. .Don’t throw gas on ramp or down grates.

. 	Don’t pee down grates.  (Toilet is at east end of Hangar G, or use own bottle).

Before Starting

All hangar gear stowed, cars inside, hangar closed, locked, side latches engaged.

All pax and luggage aboard, area clear before starting.  (Unqualified persons or

bystanders must not help with starting, must stay far away!)

Upon Return

Aim for markers on fence. Push back enough for gas truck (but no more).

182 fuel valve not on “Both” when fueling or when parked on lateral slope.

Tanks left within ½” of full (or leave down as requested by next pilot).

Placard/note fuel tach-hrs down if aircraft is left not fully fueled for any reason.

Keys in pocket before moving the prop. (Avoid moving prop if possible.)

When pushing back, stay lined up, aim tail at hanger back wall centerline.

Be prepared to instantly shut off (or lift up) the awkward, hard-to-control tug.

Prop blade roots are tender; don’t push too hard. Never push out on the blades.

Don’t push on the spinner or cowling. Main force should be on nosewheel.

Restrain helpers, don’t let them dent tender surfaces, scoop a/c docs or equipment.

Bugs off windshield, trash out of airplane, pick up.  (See shutdown checklist.)

Leave pilot’s window open.  Lock and latch hangar door.