Shutdown Checklist                                                      NR   9/9/99, rev. 2/10/02, 7/14/04

Wing flaps up, COWL FLAPS OPEN

Mixture lean for taxiing (182)

Transponder off or standby

Call for fuel, parking, ground transportation as req’d

Close flight plan, cancel IFR (if non-tower arpt), contact Customs

Pilot-controlled airport lighting -- shut off

RADIOS OFF at radio master sw.

Run engine up lean to slightly higher rpm and cut mixture off

Cold wx (and no preheat): momentarily engage starter

MASTER OFF, KEYS OUT

CONTROLS LOCKED (Gust lock on when parked, except in hanger)

ANC Headphone batteries shut off, other personal battery operated eqpt off.

TANK not on “both” (182)  during fueling, or when parked on lateral slope.

TACH TIME on ticket, elapsed dates, times if mins apply

Personal log book and aircraft day-book entries made

Status, oil additions, VOT rdgs, and maintenance items in book: placard wheel if required

Personal belongings and trash out (caution pax not to scoop A/C doc., manuals).

Airplane logs, manuals, documents, check-lists in.

Airplane aux eqpt.: fuel drain cup, phones, fire extingr, pencils, sick bags, tow-bar,

oil, cleanup kit, sunscreens, ladder (182)

Sunscreens installed (except in winter)

Towbar disengaged and stowed

Seatbelts latched and stowed

Brakes OFF (or as req’d)

Bugs wiped off canopy

Doors and baggage compartments locked

Airplane fueled or fuel status noted in book

Oil added if req'd

Tire pressure checked as req'd

Gas filler caps on, oil filler cap on

Chocks in place, AIRPLANE CHOCKED and TIED DOWN if outside

Cold wx:  Block heater plugged in

Apr-May: Bird corks installed

At BED:  Door area, wheel tracks into hangar cleared of snow and ice

BED security escort if req’d 781-869-8000 offc hrs; 617-212-6592 after hrs.

Hgr eqpt complete: snow-shovel, ice-chopper, towbar, ladder, gas-cans, ext.

cord, tire-pump.  Leaves swept out.  No gas in plastic gas offload tanks.

Hgr supplies complete: oil, plastic polish, rags, trashbags, clean wipers,

Hangar lights out, ACCESS DOOR LOCKED, MAIN DOOR LATCHED both

sides

Notify next person, maintenance, make schedule mods if req’d

MAKE ON-LINE TIMESHEET and MAINT ENTRIES